# Lab 2 Solutions: Integer Arithmetic

Use these solutions to check the work you submitted, and to understand anything that did not run as you expected.

How to Use These Solutions: - These are model answers: one correct solution per question. - Your own solution may use different registers, prompts or instruction orderings and still be fully correct. - Every solution was assembled and run in MARS 4.5. - Compare each model answer with the code you submitted. Where they differ, run both in MARS and step through them to see why. - Check your own work against these points: - your code assembles and runs in MARS, and answers the question that was asked; - you included the header comment block (Title, Author, Date, Description, Input, Output); - both the .data and .text segments are present and used properly; - you wrote a brief, in-your-own-words comment on every code line. - The key points under each solution explain the concept being tested and the mistakes that are easy to make.

# Question Q2.1

# Title: Evaluate A + 2B - 5 # Author: <First name> <Surname> # Date: 03/09/2026 # Description: Reads two integers A and B, then prints the value of A + 2B - 5. # Input: 5 7 # Output: # Enter A: Enter B: A + 2B - 5 = 14 ################## Data segment ################## .data promptA: .asciiz "Enter A: " # Prompt asking for the first integer promptB: .asciiz "Enter B: " # Prompt asking for the second integer answer: .asciiz "A + 2B - 5 = " # Label printed in front of the result newline: .asciiz "\n" # A single newline character ################## Code segment ################## .text .globl main # Mark "main" as globally visible main: # main function entry li $v0, 4 # Service 4 = print string la $a0, promptA # Load the address of the prompt for A syscall # Print "Enter A: " li $v0, 5 # Service 5 = read integer syscall # Read A into $v0 move $t0, $v0 # Copy A into $t0 so the next read cannot destroy it li $v0, 4 # Service 4 = print string la $a0, promptB # Load the address of the prompt for B syscall # Print "Enter B: " li $v0, 5 # Service 5 = read integer syscall # Read B into $v0 sll $t1, $v0, 1 # $t1 = B shifted left 1 place, so $t1 = 2B addu $t2, $t0, $t1 # $t2 = A + 2B addiu $t2, $t2, -5 # $t2 = A + 2B - 5, the required result li $v0, 4 # Service 4 = print string la $a0, answer # Load the address of "A + 2B - 5 = " syscall # Print the label li $v0, 1 # Service 1 = print integer move $a0, $t2 # Put the result into $a0 ready for printing syscall # Print the value of A + 2B - 5 li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 10 # Service 10 = exit syscall # End of program

Key points:

- Doubling B needs no multiply instruction. 
Either add $t2, $t1, $t1 or sll $t1, $t1, 1 gives 2B.

- `addi` with a negative immediate subtracts. addi $t3, $t3, -5 is the whole of the “− 5”; no separate sub is needed.

- Build the expression one step at a time, each step landing in a register.

- Beware: after reading A you must copy it out of $v0, because the next li $v0, 4 destroys it.

- Test example: A = 5 and B = 7 gives 5 + 14 − 5 = 14.

# Question Q2.2

# Title: Bitwise AND, OR, XOR and NOR # Author: <First name> <Surname> # Date: 03/09/2026 # Description: Sets $s1 = 0x12345678 and $s2 = 0xffff9a00, applies and/or/xor/nor into $s3 to $s6, and prints all four results in hexadecimal. # Input: none, the two register values are set inside the program # Output: # and $s3 = 0x12341200 # or $s4 = 0xffffde78 # xor $s5 = 0xedcbcc78 # nor $s6 = 0x00002187 ################## Data segment ################## .data msgAnd: .asciiz "and $s3 = " # Label printed in front of the AND result msgOr: .asciiz "or $s4 = " # Label printed in front of the OR result msgXor: .asciiz "xor $s5 = " # Label printed in front of the XOR result msgNor: .asciiz "nor $s6 = " # Label printed in front of the NOR result newline: .asciiz "\n" # A single newline character ################## Code segment ################## .text .globl main # Mark "main" as globally visible main: # main function entry li $s1, 0x12345678 # $s1 = 0x12345678, the first test pattern li $s2, 0xffff9a00 # $s2 = 0xffff9a00, the second test pattern and $s3, $s1, $s2 # $s3 = $s1 AND $s2, keeps only bits set in both or $s4, $s1, $s2 # $s4 = $s1 OR $s2, keeps bits set in either xor $s5, $s1, $s2 # $s5 = $s1 XOR $s2, keeps bits set in just one nor $s6, $s1, $s2 # $s6 = NOT ($s1 OR $s2), the inverse of $s4 li $v0, 4 # Service 4 = print string la $a0, msgAnd # Load the label that names this result syscall # Print the label li $v0, 34 # Service 34 = print integer in hexadecimal move $a0, $s3 # Put $s3 into $a0 ready for printing syscall # Print $s3 as an 8-digit hex number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 4 # Service 4 = print string la $a0, msgOr # Load the label that names this result syscall # Print the label li $v0, 34 # Service 34 = print integer in hexadecimal move $a0, $s4 # Put $s4 into $a0 ready for printing syscall # Print $s4 as an 8-digit hex number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 4 # Service 4 = print string la $a0, msgXor # Load the label that names this result syscall # Print the label li $v0, 34 # Service 34 = print integer in hexadecimal move $a0, $s5 # Put $s5 into $a0 ready for printing syscall # Print $s5 as an 8-digit hex number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 4 # Service 4 = print string la $a0, msgNor # Load the label that names this result syscall # Print the label li $v0, 34 # Service 34 = print integer in hexadecimal move $a0, $s6 # Put $s6 into $a0 ready for printing syscall # Print $s6 as an 8-digit hex number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 10 # Service 10 = exit syscall # End of program

Q2.2 Written answers:

| a. Content of $s3 after and $s3, $s1, $s2 (in hexadecimal): | 0x12341200 |
| --- | --- |
| b. Content of $s4 after or $s4, $s1, $s2 (in hexadecimal): | 0xffffde78 |
| c. Content of $s5 after xor $s5, $s1, $s2 (in hexadecimal): | 0xedcbcc78 |
| d. Content of $s6 after nor $s6, $s1, $s2 (in hexadecimal): | 0x00002187 |

Key points:

- `and` clears bits

- `or` sets bits

- `xor` flips bits.

  - For masking, each result only keeps the pattern of its mask.

- `nor` is how MIPS gives you NOT. There is no not instruction: nor $s1, $s2, $s2 is the idiom for inverting a single register.

Beware:

- andi, ori and xori zero-extend their 16-bit immediate. (Logic instructions)

- addi sign-extends its 16-bit immediate.  (Arithmetic instruction)

A full 32-bit constant needs li, which the assembler expands into lui + ori — so expect two machine instructions per li in the Text Segment pane.

- MARS can print hexadecimal for you: service 34 prints an integer as hex (the 0x prefix is included already), 36 prints unsigned and 35 prints binary.

- Self-check without the answer key: $s4 (or) and $s6 (nor) must OR together to 0xffffffff, because nor is the exact complement of or.

- You can work out one hex digit at a time on paper. Each hex digit is 4 bits, so each operation applies to each digit independently.

# Question Q2.3

# Title: Logical and arithmetic shifts # Author: <First name> <Surname> # Date: 03/09/2026 # Description: Sets $s1 = 0x87654321, applies sll 16, srl 8 and sra 12 into $s2 to $s4, and prints all three results in hexadecimal. # Input: none, the register value is set inside the program # Output: # sll 16 $s2 = 0x43210000 # srl 8 $s3 = 0x00876543 # sra 12 $s4 = 0xfff87654 ################## Data segment ################## .data msgSll: .asciiz "sll 16 $s2 = " # Label printed in front of the sll result msgSrl: .asciiz "srl 8 $s3 = " # Label printed in front of the srl result msgSra: .asciiz "sra 12 $s4 = " # Label printed in front of the sra result newline: .asciiz "\n" # A single newline character ################## Code segment ################## .text .globl main # Mark "main" as globally visible main: # main function entry li $s1, 0x87654321 # $s1 = 0x87654321, the test pattern (its top bit is 1) sll $s2, $s1, 16 # $s2 = $s1 shifted left 16, zeros come in from the right srl $s3, $s1, 8 # $s3 = $s1 shifted right 8, zeros come in from the left sra $s4, $s1, 12 # $s4 = $s1 shifted right 12, the sign bit 1 comes in from the left li $v0, 4 # Service 4 = print string la $a0, msgSll # Load the label that names this result syscall # Print the label li $v0, 34 # Service 34 = print integer in hexadecimal move $a0, $s2 # Put $s2 into $a0 ready for printing syscall # Print $s2 as an 8-digit hex number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 4 # Service 4 = print string la $a0, msgSrl # Load the label that names this result syscall # Print the label li $v0, 34 # Service 34 = print integer in hexadecimal move $a0, $s3 # Put $s3 into $a0 ready for printing syscall # Print $s3 as an 8-digit hex number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 4 # Service 4 = print string la $a0, msgSra # Load the label that names this result syscall # Print the label li $v0, 34 # Service 34 = print integer in hexadecimal move $a0, $s4 # Put $s4 into $a0 ready for printing syscall # Print $s4 as an 8-digit hex number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 10 # Service 10 = exit syscall # End of program

Q2.3 Written answers:

| a. Content of $s2 after sll $s2, $s1, 16 (in hexadecimal): | 0x43210000 |
| --- | --- |
| b. Content of $s3 after srl $s3, $s1, 8 (in hexadecimal): | 0x00876543 |
| c. Content of $s4 after sra $s4, $s1, 12 (in hexadecimal): | 0xfff87654 |

Key points:

- `srl` shifts in zeros  --- 0x87654321 has its top bit set, so srl by 8 gives 0x00876543

- `sra` copies the sign bit in --- sra by 12 gives 0xfff87654

The difference between the two is important here.

- `sll` by n multiplies by 2ⁿ and `srl` by n divides an unsigned value by 2ⁿ. 
Bits that get shifted “off the end”, are no longer retain in bit register.

- The shift amount is a 5-bit constant (0–31) stored inside the instruction.

- sllv, srlv and srav take the shift amount from a register.

- Shifting by a multiple of 4 moves the pattern by whole hexadecimal digits, which makes these easy to predict on paper before running them.

# Question Q2.4

# Title: Change the case of a letter # Author: <First name> <Surname> # Date: 03/09/2026 # Description: Reads one alphabetic character and prints it with its case swapped, by toggling bit 5 of its ASCII code with xori. # Input: q # Output: # Enter an alphabetic character: Case changed: Q ################## Data segment ################## .data prompt: .asciiz "Enter an alphabetic character: " # Prompt asking for one letter answer: .asciiz "Case changed: " # Label printed in front of the result newline: .asciiz "\n" # A single newline character ################## Code segment ################## .text .globl main # Mark "main" as globally visible main: # main function entry li $v0, 4 # Service 4 = print string la $a0, prompt # Load the address of the prompt syscall # Print "Enter an alphabetic character: " li $v0, 12 # Service 12 = read character syscall # Read the letter into $v0 xori $t0, $v0, 0x20 # Toggle bit 5, which flips a letter between upper and lower case li $v0, 4 # Service 4 = print string la $a0, answer # Load the address of "Case changed: " syscall # Print the label li $v0, 11 # Service 11 = print character move $a0, $t0 # Put the swapped-case letter into $a0 syscall # Print the letter li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 10 # Service 10 = exit syscall # End of program

Key points:

- The gap between `'a'` (0x61) and `'A'` (0x41) is 0x20 — a single bit, bit 5. Therefore, a case change is one bit of the ASCII code, not a separate property.

- Changing case is achieved with one instruction: `xori $t0, $v0, 0x20`. It works in both directions at once, so no branch is needed to decide which way to convert.

- Adding 0x20 converts only upper to lower, and subtracting only lower to upper. xori is what makes a single code path correct for both.

- Note: the andi $s0, $s0, 0xfd shown in the lab sheet does not do this job — clearing bit 5 would be 0xdf, and 0xfd clears bit 1 instead. Use xori 0x20.

- Read the character with service 12 and print it with service 11 — both work on a character code in a register, not on a string.

# Question Q2.5

# Title: Read one bit of an integer # Author: <First name> <Surname> # Date: 03/09/2026 # Description: Reads an integer and a bit position from 0 to 31, then prints the value of the bit at that position. # Input: 20 2 # Output: # Enter an integer: Enter a bit position (0 to 31): The value of that bit is: 1 ################## Data segment ################## .data promptN: .asciiz "Enter an integer: " # Prompt asking for the integer promptP: .asciiz "Enter a bit position (0 to 31): " # Prompt asking for the bit position answer: .asciiz "The value of that bit is: " # Label printed in front of the result newline: .asciiz "\n" # A single newline character ################## Code segment ################## .text .globl main # Mark "main" as globally visible main: # main function entry li $v0, 4 # Service 4 = print string la $a0, promptN # Load the address of the prompt for the integer syscall # Print "Enter an integer: " li $v0, 5 # Service 5 = read integer syscall # Read the integer into $v0 move $t0, $v0 # Copy the integer into $t0 before the next read li $v0, 4 # Service 4 = print string la $a0, promptP # Load the address of the prompt for the bit position syscall # Print "Enter a bit position (0 to 31): " li $v0, 5 # Service 5 = read integer syscall # Read the bit position into $v0 srlv $t1, $t0, $v0 # Shift the integer right by that many places, wanted bit is now bit 0 andi $t1, $t1, 1 # Keep only bit 0, so $t1 is now either 0 or 1 li $v0, 4 # Service 4 = print string la $a0, answer # Load the address of "The value of that bit is: " syscall # Print the label li $v0, 1 # Service 1 = print integer move $a0, $t1 # Put the bit value into $a0 syscall # Print the bit value li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 10 # Service 10 = exit syscall # End of program

Key points:

- `srlv` takes its shift amount from a register, which is what you need when the bit position comes from the user. The constant-amount srl cannot do this.

- Bring the wanted bit down to position 0, then mask with `andi $t2, $t2, 1`. Masking without shifting first leaves the bit at its original weight (8, not 1).

- The alternative is to build the mask instead: sllv a 1 up to the position and and it with the number. Either is correct; only the shape of the answer differs.

- A shift instruction uses only the low 5 bits of the amount, so a position of 32 silently behaves like 0 rather than reporting an error. Validate the range yourself if you want to reject it.

- Check: 20 is 10100, so bit 2 is 1 and bit 3 is 0.

# Question Q2.6

# Title: Multiply a signed number by 24.5 # Author: <First name> <Surname> # Date: 03/09/2026 # Description: Reads a signed integer x and prints 24.5*x using only shifts and adds, as (x<<4)+(x<<3)+(x>>1). The half is an arithmetic shift right, so an odd x is rounded down towards minus infinity: 5 gives 122 and -5 gives -123. # Input: 5 # Output: # Enter a signed number: The number times 24.5 is: 122 ################## Data segment ################## .data prompt: .asciiz "Enter a signed number: " # Prompt asking for the number answer: .asciiz "The number times 24.5 is: " # Label printed in front of the result newline: .asciiz "\n" # A single newline character ################## Code segment ################## .text .globl main # Mark "main" as globally visible main: # main function entry li $v0, 4 # Service 4 = print string la $a0, prompt # Load the address of the prompt syscall # Print "Enter a signed number: " li $v0, 5 # Service 5 = read integer syscall # Read the signed number x into $v0 sll $t0, $v0, 4 # $t0 = x shifted left 4 places, so $t0 = 16x sll $t1, $v0, 3 # $t1 = x shifted left 3 places, so $t1 = 8x sra $t2, $v0, 1 # $t2 = x shifted right 1 place keeping the sign, # .. so $t2 = x/2 rounded down addu $t3, $t0, $t1 # $t3 = 16x + 8x, which is 24x addu $t3, $t3, $t2 # $t3 = 24x + x/2, which is 24.5x li $v0, 4 # Service 4 = print string la $a0, answer # Load the address of "The number times 24.5 is: " syscall # Print the label li $v0, 1 # Service 1 = print integer move $a0, $t3 # Put the scaled result into $a0 syscall # Print the result li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 10 # Service 10 = exit syscall # End of program

Key points:

- 24.5x = 16x + 8x + x/2, which is (x << 4) + (x << 3) + (x >> 1). Splitting a constant into powers of two is how multiplication is done before mult is introduced.

- Use `sra` for the halving, not `srl`. On a negative value srl shifts a zero into the sign position and turns the number into a large positive one.

- Counterintuitive: `sra` rounds towards minus infinity, it does not truncate. -5 >> 1 gives -3, not -2. So this solution prints:
5 → 122 (from 122.5), and
-5 → -123 (from −122.5).
Reasoning “−122.5 truncates to −122” gives a different answer and makes correct code look wrong.

- The result is an integer: the .5 only contributes a whole number when x is even. That is expected, because this lab has no floating-point instructions.

# Question Q2.7

# Title: Swap odd and even bits # Author: <First name> <Surname> # Date: 03/09/2026 # Description: Reads an unsigned number and prints the value obtained by swapping every even-numbered bit with its odd-numbered neighbour, for example 9 becomes 6. # Input: 9 # Output: # Enter an unsigned number: After swapping the bits: 6 ################## Data segment ################## .data prompt: .asciiz "Enter an unsigned number: " # Prompt asking for the number answer: .asciiz "After swapping the bits: " # Label printed in front of the result newline: .asciiz "\n" # A single newline character ################## Code segment ################## .text .globl main # Mark "main" as globally visible main: # main function entry li $v0, 4 # Service 4 = print string la $a0, prompt # Load the address of the prompt syscall # Print "Enter an unsigned number: " li $v0, 5 # Service 5 = read integer syscall # Read the number into $v0 li $t0, 0x55555555 # $t0 = mask with a 1 in every even-numbered bit and $t1, $v0, $t0 # $t1 = only the even-numbered bits of the number sll $t1, $t1, 1 # Move those bits up one place, into the odd positions srl $t2, $v0, 1 # Move every bit of the number down one place and $t2, $t2, $t0 # $t2 = the old odd-numbered bits, now in the even positions or $t3, $t1, $t2 # Join the two halves to give the fully swapped number li $v0, 4 # Service 4 = print string la $a0, answer # Load the address of "After swapping the bits: " syscall # Print the label li $v0, 36 # Service 36 = print integer as unsigned move $a0, $t3 # Put the swapped number into $a0 syscall # Print the swapped number li $v0, 4 # Service 4 = print string la $a0, newline # Load the address of the newline syscall # Print a newline li $v0, 10 # Service 10 = exit syscall # End of program

Key points:

- `0x55555555` selects the even-numbered bits and `0xAAAAAAAA` the odd ones. In binary these are the alternating patterns 0101… and 1010…

- The whole swap is one expression:
((x & 0x55555555) << 1) | ((x >> 1) & 0x55555555).
Every pair of bits exchanges at the same time — there is no loop over the 32 positions.

- Mask after the right shift, not before, so the bit leaving position 0 cannot reappear where it does not belong. Use srl so a set top bit is not smeared through the result.

- Watch out: service 5 reads a signed integer, so a true unsigned value such as 4294967295 is rejected at run time. The result can still exceed the signed range, which is why printing with service 36 (unsigned) is safer than service 1 here.

- Check: 9 is 1001, and swapping each pair gives 0110, which is 6.

# Self-Assessment

Run your own submitted code in MARS and tick each item you can confirm. Any item you cannot tick points you to the Key points for that question.

| Q2.1 — entering 5 and 7 prints 14. | [ ] |
| --- | --- |
| Q2.2 — my four register values match the answer key above. | [ ] |
| Q2.3 — my three register values match the answer key above. | [ ] |
| Q2.4 — entering q prints Q, and entering Q prints q. | [ ] |
| Q2.5 — entering 20 and bit position 2 prints 1; bit position 3 prints 0. | [ ] |
| Q2.6 — entering 5 prints 122, and my Description says how I handled the .5 and negative values. | [ ] |
| Q2.7 — entering 9 prints 6. | [ ] |
| My header comment block is complete in every answer (Title, Author, Date, Description, Input, Output). | [ ] |
| Every line of my code carries a brief comment written in my own words. | [ ] |

What I need to revise before Lab 3:

| Questions where my program did not produce the expected output, and what was wrong with it: | Type your answer here. |
| --- | --- |
| Concepts from the Key points that I want to go over again: | Type your answer here. |