# Lab 2: Integer Arithmetic

| Student Name | Qimin Zhu | Student ID | 6888143 |
| --- | --- | --- | --- |

Objectives:

- Get familiar with the basic MIPS integer arithmetic and logic instructions.

- Use the integer addition and subtraction, bitwise logic, and shift instructions.

- Learn some useful applications of these instructions.

Outline:

- Read through the MIPS Lab 2 - Integer Arithmetic sheet (PDF).

  - Read the Code Answer Template (Page 2; below).
Each code answer should follow this template.

  - Answer questions Q2.1 – Q2.7 -- In-Lab Tasks from Section 3.5.

How to Complete This Worksheet: - Complete this worksheet individually and submit your own copy. - Write and test each answer in MARS first. - Assemble (F3) - Run (F5) - Check the output in the Run I/O window. - Prepare each answer using the Code Answer Template (page 2), then copy-paste your final working code into the grey answer box for each question. - Comment every code line in your own words, very briefly, describing what that line does. - Your code answers should be complete and runnable in MARS. - Grading scores are given to assembling code that runs in MARS. - Include the header comments, - .data segments - .text segments, - comments to show your understanding to get a score, if there’s a problem assembling your code.

# Code Answer Template

Answer each Lab question as a complete MIPS program in the format below.

Steps:

- Open the MARS editor tab.

  - Fill in the header information lines:

    - Title,

    - Author (your name and student ID),

    - Date,

    - Description

    - Input / Output of a test run.

  - Declare your strings and variables in the .data segment

  - Write your instructions in the .text segment after main:.

  - Exit your program with system call: e.g.

    - li $v0, 10

    - syscall

- Make sure your code assembles, runs, and answers the question.

- Comment each line of code with #, in your own words, briefly, e.g.

  - move $t0, $v0  # Copies $v0's value into register $t0.

- Paste your working code into the answer box.

  - We grade according to whether you answered the question, and your code assembles and runs in MARS.

# Title: [Program title, e.g. Echo an Integer] # Author: [Your Name] [Your Student ID] # Date: [DD/MM/YYYY] # Description: [One sentence: what does the program do?] # Input: [The test input you used, e.g. 20] # Output: [The exact output MARS printed, e.g.] # Enter an integer: 20 # You entered: 20 ################## Data segment ################## .data prompt: .asciiz "Enter an integer: " # Your strings / variables here result: .asciiz "You entered: " ################## Code segment ################## .text .globl main # mark the “main” function as globally visible. main: # main function entry li $v0, 4 # Service 4 = print string la $a0, prompt # Load the address of the prompt text syscall # Print the prompt li $v0, 5 # Service 5 = read integer syscall # Read the integer into $v0 move $t0, $v0 # Copy the input value into $t0 li $v0, 4 # Service 4 = print string la $a0, result # Load the address of the result text syscall # Print the result text li $v0, 1 # Service 1 = print integer move $a0, $t0 # Put the input value in $a0 syscall # Print the value # Your code may continue here ... # Always end your program with the exit system call ...

# Part 1 - Lab Tasks (Lab Sheet Section 3.5)

Answer every question using the Code Answer Template.

Each answer box should contain one complete, runnable, commented MIPS program.

## Question Q2.1

Question / Instruction Q2.1 Write a program that asks the user to enter two integers A and B, and then displays the result of computing the expression: A + 2B - 5. Hint: - Build the expression one step at a time: Comment your code, complete the code template format given.

Paste your complete MIPS code answer here Comment your code, complete the code template format given. # Title: Question 2.1 # Author: Qimin Zhu 6888143 # Date: 12/09/2026 # Description: Calculate A + 2B - 5 # Input: Two integers A and B # Output: The result of A + 2B - 5 ################## Data segment ################## .data promptA: .asciiz "Enter an integer A: " promptB: .asciiz "Enter an integer B: " ################## Code segment ################## .text .globl main main: # Print prompt A li $v0, 4 la $a0, promptA syscall # Read integer A li $v0, 5 syscall move $t0, $v0 # Print prompt B li $v0, 4 la $a0, promptB syscall # Read integer B li $v0, 5 syscall move $t1, $v0 # Calculate A + 2B - 5 add $t2, $t1, $t1 sub $t2, $t2, 5 add $t3, $t0, $t2 # Print result li $v0, 1 move $a0, $t3 syscall

## Question Q2.2

Question / Instruction Q2.2 Assume that $s1 = 0x12345678 and $s2 = 0xffff9a00. Determine the content of registers $s3 to $s6 after executing the instructions listed below. Write a program to execute these instructions and verify the content of registers $s3 to $s6. Record your predicted values in the written-answers table, then paste your verification program in the answer box. and $s3, $s1, $s2 or $s4, $s1, $s2 xor $s5, $s1, $s2 nor $s6, $s1, $s2 Hint: - Calculate the four values on paper before you write the program (write answers below). - Work one hexadecimal digit at a time: a hex digit is 4 bits, and each operation applies to each digit on its own. - Service 34 prints an integer as hexadecimal, so no conversion loop is needed.

Q2.2 Written answers:

| a. and $s3, $s1, $s2 (in hexadecimal): | $s3 = 0x1233f078 |
| --- | --- |
| b. or $s4, $s1, $s2 (in hexadecimal): | $s4 = 0xffffde78 |
| c. xor $s5, $s1, $s2 (in hexadecimal): | $s5 = 0xedcbcc78 |
| d. nor $s6, $s1, $s2 (in hexadecimal): | $s6 = 0x00002187 |
| Paste your complete MIPS code answer here # Title: Question 2.2 # Author: Qimin Zhu 6888143 # Date: 12/09/2026 # Description: Perform addition, OR, XOR, and NOR operations on two hexadecimal integers # Input: A = 0x12345678, B = 0xFFFF9A00 # Output: Results of A + B, A OR B, A XOR B, and A NOR B ################## Data segment ################## .data promptA: .asciiz "Enter an integer A: " promptB: .asciiz "Enter an integer B: " newline: .asciiz "\n" ################## Code segment ################## .text .globl main main: # Load integer A li $s1, 0x12345678 # Load integer B li $s2, 0xFFFF9A00 # Calculate addition add $s3, $s1, $s2 # Print result li $v0, 34 move $a0, $s3 syscall # Print newline li $v0, 4 la $a0, newline syscall # Calculate OR or $s4, $s1, $s2 # Print result li $v0, 34 move $a0, $s4 syscall # Print newline li $v0, 4 la $a0, newline syscall # Calculate XOR xor $s5, $s1, $s2 # Print result li $v0, 34 move $a0, $s5 syscall # Print newline li $v0, 4 la $a0, newline syscall # Calculate NOR nor $s6, $s1, $s2 # Print result li $v0, 34 move $a0, $s6 syscall |  |

## Question Q2.3

Question / Instruction Q2.3 Assume that $s1 = 0x87654321. Determine the content of registers $s2 to $s4 after executing the instructions listed below. Write a program to execute these instructions and verify the content of registers $s2 to $s4. Record your predicted values in the written-answers table, then paste your verification program in the answer box. sll $s2, $s1, 16 srl $s3, $s1, 8 sra $s4, $s1, 12 Hint: - The difference between srl and sra is the key to this question. - srl shifts zeros in at the top, while sra copies the sign bit in. Note that 0x87654321 already has its top bit set (8 = 1000 in 4-bits). - Shifting by a multiple of 4 moves the pattern by a whole hexadecimal digit, so these can be worked out on paper. Comment your code, complete the code template format given.

Paste your complete MIPS code answer here # Title: Question 2.3 # Author: Qimin Zhu 6888143 # Date: 12/09/2026 # Description: Perform SLL, SRL, and SRA shift operations on a hexadecimal integer # Input: A = 0x87654321 # Output: Results of SLL by 16, SRL by 8, and SRA by 12 ################## Data segment ################## .data promptA: .asciiz "Enter an integer A: " promptB: .asciiz "Enter an integer B: " newline: .asciiz "\n" ################## Code segment ################## .text .globl main main: # Load integer A li $s1, 0x87654321 # Calculate SLL sll $s2, $s1, 16 # Print result li $v0, 34 move $a0, $s2 syscall # Print newline li $v0, 4 la $a0, newline syscall # Calculate SRL srl $s3, $s1, 8 # Print result li $v0, 34 move $a0, $s3 syscall # Print newline li $v0, 4 la $a0, newline syscall # Calculate SRA sra $s4, $s1, 12 # Print result li $v0, 34 move $a0, $s4 syscall

Q2.3 Written answers:

| a. sll $s2, $s1, 16 (in hexadecimal): | $s2 = 0x43210000 |
| --- | --- |
| b. srl $s3, $s1, 8 (in hexadecimal): | $s3 = 0x00876543 |
| c. sra $s4, $s1, 12 (in hexadecimal): | $s4 = 0xfff87654 |

## Question Q2.4

Question / Instruction Q2.4 Write a program that asks the user to enter an alphabetic character (either lower or upper case), changes the case of the character from lower to upper and from upper to lower, and displays it. Hint: - Compare 'a' (0x61) with 'A' (0x41): they differ by 0x20, which is a single bit. - One logic instruction can flip that bit in both directions at once. - Read a character with service 12 and print one with service 11; both work on a character code in a register. - Syscall Documentation: https://dpetersanderson.github.io/Help/SyscallHelp.html Comment your code, complete the code template format given.

Paste your complete MIPS code answer here # Title: Question 2.4 # Author: Qimin Zhu 6888143 # Date: 12/09/2026 # Description: Toggle the case of an input letter using XOR # Input: One letter # Output: The input letter with its uppercase/lowercase case toggled ################## Data segment ################## .data promptA: .asciiz "Enter a letter: " newline: .asciiz "\n" space: .asciiz " to " ################## Code segment ################## .text .globl main main: # Print the prompt li $v0, 4 la $a0, promptA syscall # Read a letter from input li $v0, 12 syscall move $s1, $v0 # Load the case-toggle mask li $s0, 0x20 # Toggle the letter case using XOR xor $s2, $s1, $s0 # Print separator li $v0, 4 la $a0, space syscall # Print the result li $v0, 11 move $a0, $s2 syscall

## Question Q2.5

Question / Instruction Q2.5 Write a program that asks the user to enter an integer number and reads it. Then ask the user to enter a bit position (between 0 and 31) and display the value of that bit. Hint: - Bring the wanted bit down to position 0, then mask off everything else. - The position comes from the user, so the shift amount is in a register. - Use srlv, (not srl), which allows you to shift by a register (variable) amount (from the user’s input). - andi with 1 then keeps just the lowest bit (of a number). Comment your code, complete the code template format given.

Paste your complete MIPS code answer here # Title: Question 2.5 # Author: Qimin Zhu 6888143 # Date: 12/09/2026 # Description: Read an integer and display the value of a specified bit # Input: An integer and a bit position (0-31) # Output: The value of the specified bit (0 or 1) ################## Data segment ################## .data promptA: .asciiz "Enter an integer: " promptB: .asciiz "Enter a num (0-31): " newline: .asciiz "\n" ################## Code segment ################## .text .globl main main: # Print prompt for the integer li $v0, 4 la $a0, promptA syscall # Read an integer li $v0, 5 syscall move $s0, $v0 # Print prompt for the bit position li $v0, 4 la $a0, promptB syscall # Read the bit position li $v0, 5 syscall move $s1, $v0 # Shift the wanted bit to position 0 srlv $s2, $s0, $s1 # Keep only the lowest bit andi $s2, $s2, 1 # Print the result li $v0, 1 move $a0, $s2 syscall

## Question Q2.6

Question / Instruction Q2.6 Write a program that asks the user to enter a signed number and reads it. Then display the result of multiplying this number by 24.5. Hint: - For 24.5, you will need to make use of - multiplication’s distributivity (as mentioned in the Lab PDF) using shifts. - sra to shift 1 place to the right to calculate x/2 (which keeps the sign). Comment your code, complete the code template format given.

Paste your complete MIPS code answer here # Title: Question 2.6 # Author: Qimin Zhu 6888143 # Date: 12/09/2026 # Description: Multiply a signed integer by 24.5 using shifts # Input: A signed integer # Output: The integer result of 24.5 × input ################## Data segment ################## .data promptA: .asciiz "Enter an integer: " newline: .asciiz "\n" ################## Code segment ################## .text .globl main main: # Print prompt for the integer li $v0, 4 la $a0, promptA syscall # Read a signed integer li $v0, 5 syscall move $s0, $v0 # Calculate x × 16 sll $s1, $s0, 4 # Calculate x × 8 sll $s2, $s0, 3 # Calculate x / 2 sra $s3, $s0, 1 # Calculate 16x + 8x add $s4, $s1, $s2 # Calculate 24x + x/2 add $s5, $s4, $s3 # Print the result li $v0, 1 move $a0, $s5 syscall

## Question Q2.7

Question / Instruction Q2.7 w Write a program that asks the user to enter an unsigned number and reads it. - Then swap the bits at odd positions with those at even positions and display the resulting number. Hints: - User enters 9 Output 6 - 1001 0110 - Take the approach of mask, shift, mask, shift, combine, no need for looping or branching. - Split the number into two halves, move each half, then join them back together. - Selecting the even positions (bits 0, 2, 4, …): - You need a value that has a 1 in every even position and a 0 elsewhere. - Write that pattern out in binary, four bits at a time — …0101 0101 0101 — and convert each group of four to one hex digit. - and your number with it. - Selecting the odd positions (bits 1, 3, 5, …): - This is the complement of the even pattern. Comment your code, complete the code template format given.

Paste your complete MIPS code answer here # Title: Question 2.7 # Author: Qimin Zhu 6888143 # Date: 12/09/2026 # Description: Swap the bits at odd and even positions # Input: An unsigned integer # Output: The integer with odd and even bits swapped ################## Data segment ################## .data promptA: .asciiz "Enter an integer: " newline: .asciiz "\n" ################## Code segment ################## .text .globl main main: # Print prompt for the integer li $v0, 4 la $a0, promptA syscall # Read an integer li $v0, 5 syscall move $s0, $v0 # Load masks li $s1, 0x55555555 # Mask for even positions li $s2, 0xAAAAAAAA # Mask for odd positions # Extract even-position bits and shift them to odd positions and $s3, $s0, $s1 sll $s3, $s3, 1 # Extract odd-position bits and shift them to even positions and $s4, $s0, $s2 srl $s4, $s4, 1 # Combine the shifted bits or $s5, $s3, $s4 # Print the result li $v0, 1 move $a0, $s5 syscall

Congratulations!

You have used the MIPS integer arithmetic, bitwise logic and shift instructions, predicted register contents and verified them in MARS, changed the case of a character with logic instructions, extracted a single bit, and multiplied using shifts and additions.

Save this worksheet as PDF, add your Student ID into the filename and upload it to the Assignment page on MyCourses.